# SPAT/ATSPM Comparison Algorithm

- Identify SPaT Event State Transitions

SPaT Message Selection and Ordering

- For the defined aggregation period, identify all received SPaT messages for the signalized intersection and place in a list.

- Sort the list of selected SPaT messages in ascending order based on the message production timestamp.

Event State Extraction

- For each SPaT message, parse and extract the event state information associated with each signal group contained in the message.

Event State Transition Detection

- For each signal group, an event state transition is identified when the event state for the signal group in one SPaT message differs from the event state for that same signal group in the immediately preceding SPaT message.

- Record each detected transition in an event state transition list, preserving associated time and signal group details.

Signal Head Indication Derivation and Filtering

- For each event state transition in the transition list, determine the corresponding signal head indication (e.g., red, yellow, green) based on the event state associated with the transition as shown in the table. Note: some event state transitions are not intended to be evaluated, and are removed, per this table.

| Event State | Signal head indication |
| --- | --- |
| (0) unavailable | Remove from list |
| (1) dark | Remove from list |
| (2) stop-Then-Proceed | Red |
| (3) stop-And-remain | Red |
| (4) pre-Movement | Remove from list |
| (5) permissive-Movement-Allowed | Green |
| (6) protected-Movement-Allowed | Green |
| (7) permissive-Clearance | Yellow |
| (8) protected-Clearance | Yellow |
| (9) caution-Conflicting-Traffic | Remove from list |

- Identify ATSPM Signal Indication Transitions

ATSPM Event Log Selection and Ordering

- During the defined aggregation period, obtain all ATSPM event log records for the signalized intersection

- Order the selected ATSPM event logs in ascending order based on the event production timestamp.

- All timestamps are initially assumed to be in local time. Convert timestamps of the ATSPM event logs from local time to Coordinated Universal Time (UTC).

Event Type Filtering

- Only retain records with event code types 1, 8, and 10

Signal indication Transition Derivation

- Determine the signal indication associated with the transition as follows:

  - Event code 10 (Begin Red Clearance)→  Red

  - Event code 8 (Begin Yellow Clearance)→ Yellow

  - Event code 1 (Begin Green) → Green

Determine which phase the transition is for

- The phase associated with the transition is for is indicated in the ATSPM “parameter” field

- Determine SPaT Signal Group and ATSPM Phase Pairings

By default, all SPaT event state transitions are paired to ATSPM signal indication transitions by matching values between the SPaT signal group and the ATSPM phase.

However, there may be situations where signal group values are not designed to match exactly with phase values, or SPaT event state transitions are not directly comparable with ATSPM phase transitions. To account for these cases, the user is able to override this default pairing by entering values as indicated below.

- Signal Group (SPaT) number

- Primary Phase (ATSPM) number

- Secondary Phase (ATSPM) number  - optional.

If the secondary phase (optional) parameter is not specified, SPaT event state transitions for the configured SPaT signal group are paired with ATSPM signal indication transitions that are associated with the ATSPM phase indicated in the configured primary phase.

If both a primary phase (ATSPM) and secondary phase (ATSPM) are specified in the configuration, a process is applied to derive a resultant list of signal indication transitions (that represents the combined behavior of the primary and secondary phases) that is then paired with the SPaT event state transitions from the configured SPaT signal group. The resultant ATSPM transition is generated per the following rules:

- Include all signal head transitions (ATSPM) from the specified primary and secondary phases into a single list, in time order.

- If a transition in the primary phase is Green, then the resultant indication is green.

- If a transition in the primary phase is Yellow, then the resultant indication is yellow.

- If a transition in the primary phase is Red, then the resultant is the most recent transition in the secondary phase

- A transition in the secondary phase is indicated at the resultant, only if the most recent transition of the primary phase is red. Otherwise, it is not included in the resultant list.

An example of the application of these rules is provided in the table below:

| Primary/Secondary | Signal Transition |  | Resultant Transition |
| --- | --- | --- | --- |
| Primary | Green |  | Green |
| Secondary |  | Green | - |
| Primary | Yellow |  | Yellow |
| Primary | Red |  | Green |
| Secondary |  | Yellow | Yellow |
| Secondary |  | Red | Red |
| Secondary |  | Green | Green |
| Secondary |  | Yellow | Yellow |
| Secondary |  | Red | Red |
| Primary | Green |  | Green |
| Primary | Yellow |  | Yellow |
| Primary | Red |  | Red |
| Secondary |  | Green | Green |
| Secondary |  | Yellow | Yellow |

- Comparing paired SPaT Event State transitions and ATSPM signal indication transitions

The system first determines whether each SPaT signal group can be meaningfully paired with ATSPM data. If a signal group appears in SPaT messages, but does not correspond to any ATSPM event parameter and is not explicitly defined in the user configuration, the system generates a warning indicating that the SPaT signal group cannot be paired. This warning is intended for diagnostic and informational purposes and is not necessarily an indication of system issues.

For signal groups that are comparable with ATSPM data (either through default pairing rules or through user-specified configuration) the system compares SPaT event state transitions against the corresponding ATSPM signal indication transitions.

The comparison is performed separately for each signal indication (green, yellow, and red). For a given signal group, each SPaT transition of a particular indication is evaluated against ATSPM transitions of the same indication. A SPaT transition is considered paired if a matching ATSPM transition occurs within a ±3-second time window centered on the SPaT transition timestamp. If no ATSPM transition of the same indication is found within this window, the SPaT transition is considered unpaired. This same temporal pairing logic is applied consistently to green, yellow, and red transitions.

After all transitions for a signal group have been evaluated, the system calculates the percentage of paired transitions for each indication type. If, for any indication (green, yellow, or red), the percentage of paired transitions falls below 90 percent, the system generates an error indicating insufficient agreement between the SPaT and ATSPM data for that signal group. The error includes the time, intersection information, the signal group, the signal indication, and the paired percentage.